Introduction: System Analysis

https://ctms.engin.umich.edu/CTMS/index.php?example=Introduction&section=SystemAnalysis

https://ctms.engin.umich.edu/CTMS/index.php?example=Introduction&section=ControlPID

- Dynamic system

- A mass, spring and damper system

Md2xdt2+bdxdt+kx =F

- R,L,C circuit

L d2idt2+Rdidt+1Ci=dEdt

- Basic terminology

  - Time response

- General solution :

xgt=xht+xp(t)

- Transient response

xgt, t<∞,  :  depends on initial conditions

- Steady State response

xgt, t=∞,  :  depends on system inputs

- Domain : time

  - Frequency response

Linear Time Independent system

In the steady state, if a input is a sinusoidal (magnitude, frequency, phase)

- Output has same frequency but different magnitude and phase of the input.

- Laplace transform

xt= 0∞xtestdt

Time domain  frequency domain

- In the steady state

s-→jω, ω:the angular frequency

%% radian

- π: 3.1415…., a ir-rational number

- the length of a circle with r radius = 2πr

- the relation between radian and angle

2πradian=360 (degree)

- the frequency, f =1T,  1time=Hz , where T :a period

ft+T =ft, T>0

- the angular frequency ω :[radiantime]

2πf=ω

Examp;  50Hz  sin wave

Asin2πft=Asinωt=Asin(2*3.14*50 t)

- Transfer function

Ts=outputsinputs , s=laplace "s"

  - Stability

Bounded input bounded output

- Frequency domain

The poles of the transfer function are in LHP in “s”

Ex. G(s)=1s2+2s+5

Poles of G(s) are all in the Left Half Plane (LHP) in the complex domain

- Poles of G(s)  are all in LHP  BIBO Stable

  - First Order system

- The transfer function

Gs=bs+a or kdcτs+1

- DC gain kdc

The ratio of the mag. Of the steady state output to the magnitude of the step input

%% The final value theorem

limt→∞yt= lims→0sY(s)

The unit step has the Laplace as

Lstept= 0∞e-st1dt =1s

%%                           The output at the steady state is

limt→∞yt= lims→0sY(s)= lims→0s1sbs+a=ba-→DC gain

- Time constant Tc with step input

The first order system time constant is Tc=τ=1a , where

- With zero initial condition

yTc=0.63 yss

- With no-input

yTc=0.37 y0

Step-response

- Settling time Ts : 90%

- Rise time  Tr: from 10 to 90%

- Bode Plot

20log10Y

%%%

Gs=510s+1

In the lower range of frequencies,

20 log105 ~14

%%%%

  - Second order system

- Governing equation

my+by+ky=u

y+2ζωny+ωn2y=kdcωn2u

The transfer function

Gs=1ms2+bs+k

- Terminologies

-DC gain kdc=1k

-Damping ratio  ζ=b2km

-Natural frequency ωn= km

- Under damped System :  ζ<1

- Overdamped ζ>1

- Critically damped ζ=1

- Undamped ζ=0

- Bode plot

- Dynamic  System

x=dxdt=fx,u,t

- Time invariant:

x=dxdt=fx,u

- Linear Time Invariant

x=dxdt=Ax+Bu

- State space representation

x= Ax+Bu

y=Cx+Du

- State variables x(t)

- Input control vector u(t)

- The system matrix A, Input matrix B, Output matrix C, feedforward matrix D

- Transfer Function Representation

- Laplace transform

Fs= Lft= 0∞e-stftdt

- Property

Ldnfdtn=snFs-sn-1f0-…-fn-1(0)

- The frequency domain method

andnydtn+…+a0y=bmdmudtm+…+b0u

Using the property,

ansnYs+…+a0Ys=bmsmUs+…+b0U(s)

If all the initial condition are zero

- The transfer function G(s)

Gs=YsUs= bmsm+…+b0ansn+…+a0
=NsDs=K s-z1…(s-zn)s-p1…(s-pn)

- System poles , zeros

- System gain =bman

- Equivalence to the state space

Gs=YsUs=CSI-A-1B+D

- Example – Electrical System

  - Governing equation

Vt-Ri-Ldidt-1Cidt=0

Remember

q= idt

- Define the state

x= qi

- The state space representation

x= ididt= 01-1LC-RLqi+ 01LV

The output we may choose as

y=0 1qi

- The transfer function is

IsVs=CSI-A-1B+D=01s1001- 01-1LC-RL-101/L

-→IsVs= sLs2+Rs+1/C